Supplemental information

Determination of liraglutide in rat plasma using selective liquid chromatography–tandem mass spectrometry

Younah Kim1, Hyoje Jo1, Yelim Lee2, Soo Yeon Kang2, Sangkyu Lee3,*

1College of Pharmacy, Kyungpook National University, Daegu 41566, Republic of Korea

2 DK Medicine Research Institute, Dongkook Pharmaceutical, Yongin 16954, Republic of Korea

3School of Pharmacy, Sungkyunkwan University, Suwon 16419, Republic of Korea

*Corresponding Author: Sangkyu Lee Ph.D., Associate Professor, School of Pharmacy, Sungkyunkwan University, Suwon 16419, Republic of Korea, Tel.: +82-31-290-7708 FAX: +82-292-8800

Legend of supplemental data

Table S1. LC-MS/MS conditions for analytes

Table S2. Stability results for of validation method for liraglutide in rat plasma

Figure S1. MS/MS spectra of (A) liraglutide and (B) exenatide as internal standard (IS).

Figure S2. Representative SRM chromatogram of liraglutide (●) and column carryover (○, second to bottom). The transitions from precursor ion to product ion are m/z 938.6→135.9 for liraglutide. Column carryover was measured by detection liraglutide peaks after injecting 500 ng/mL of liraglutide. (A) Mobile phase A is Water with 0.01% formic acid and mobile phase B is ACN with 0.01% formic acid (FA). (B) mobile phase A is Water with 0.3% formic acid and mobile phase B is ACN (50%) and MeOH (50%) with 0.3% formic acid.

Figure S3. Calibration curve overlay for liraglutide over the course of 3 days. Eight calibration standards with concentrations of 5, 10, 25, 50, 100, 250, 400, and 600 ng/mL for liraglutide.

Figure S1. MS/MS spectra of (A) liraglutide and (B) exenatide as internal standard (IS).

Figure S2. Representative SRM chromatogram of liraglutide (●) and column carryover (○, second to bottom). The transitions from precursor ion to product ion are m/z 938.6→135.9 for liraglutide. Column carryover was measured by detection liraglutide peaks after injecting 500 ng/mL of liraglutide. (A) Mobile phase A is Water with 0.01% formic acid and mobile phase B is ACN with 0.01% formic acid (FA). (B) mobile phase A is Water with 0.3% formic acid and mobile phase B is CAN (50%) and MeOH (50%) with 0.3% formic acid.

| Instrument |  | Thermo Vanquish |  |
| --- | --- | --- | --- |
| Column |  | ACQUITY PREMIER BEH C18, 2.1 mm X 150 mm |  |
| Flow Rate |  | 0.25 mL/min |  |
| Auto sampler temperature |  | 10℃ |  |
| Column temperature |  | 80℃ |  |
| Mobile phase |  | (A) Water with 0.3 % formic acid (B) ACN/MeOH = 50/50 (v/v) with 0.3% formic acid |  |
| Run time |  | 8 min |  |
| Injection volume |  | 20 L |  |
| Instrument |  | Thermo TSQ Vantage |  |
|  |  | Precursor Ion | Product Ion |
| Standard | Liraglutide | 938.6 | 135.9 |
|  |  | 1251.5 | 208.9 |
|  | Exenatide (IS) | 1047.6 | 1264.4 |

Table S1. LC-MS/MS conditions for analytes

Figure S3. Calibration curve overlay for liraglutide over the course of 3 days. Eight calibration standards with concentrations of 5, 10, 25, 50, 100, 250, 400, and 600 ng/mL for liraglutide.

Table S2. Stability results for of validation method for liraglutide in rat plasma

| Stability condition | Nominal concentration (ng/mL) |  |  |
| --- | --- | --- | --- |
|  | 25 | 100 | 400 |
| Stock solution |  |  |  |
| 30 days, -20°C (%) | 94.66 | 98.06 | 96.65 |
| Plasma samples |  |  |  |
| Freeze-thaw (3 cycles) (%) | 104.67 | NA | 92.50 |
| 6 h, room temperature (%) | 99.84 | NA | 101.45 |
| 15 days, -80°C (%) | 97.84 | NA | 103.60 |

NA, not applicable